SNMP Stands for Simple Network Management Protocol. SNMP was developed to allow the administrator to monitor networking equipment like Routers, Switches, Firewalls, Servers, CCTV Cameras and Wireless devices. Also to modify setting and configurations on Networking equipment.

Initially SNMP was developed to monitor routers and Switches only and now we can monitor many Networking equipment.

It is an application layer protocol. It uses UDP port no 161 and 162.

Organization uses SNMP protocol to monitor and manage devices on local area network that is LAN or wide area network that is WAN.

Most networking devices come with SNMP agents built-in. If not, some systems allow administrators to install or enable an SNMP agent manually.

There are two components of SNMP protocol:
1. SNMP manager
2. SNMP client

The SNMP Manager is the central system used to monitor an SNMP-enabled network, also known as a Network Management Station (NMS).

The SNMP Manager is responsible for communicating with SNMP agents and runs on a host within the network.

The SNMP agent is software running on a network device that provides status and performance information to the SNMP manager.

There are 2 terms in SNMP:
1. MIB – Management Information Base
2. OID – Object Identifier

Each management station or agent in an SNMP managed network maintain a local database of
information relevant to network management is known as management information based.

OID is used to identify managed object in a MIB.

Snmp agent configuration: 
snmp-server community read ro
snmp-server community write rw